# Initial Research

## What is Network Traffic Analysis?

A method of tracking network activity in order to identify issues with security and operations.

## Monitoring - Harvey:

Packet sniffing implementation

https://www.uv.mx/personal/angelperez/files/2018/10/sniffers_texto.pdf

A packet sniffer is consisted of two main parts:

- A network adapter - this connects the sniffer to the existing network

- Software that provides a way to log, see or analyse data.

Not all packets take the same route so the network adapter needs to change that network so that all packets pass through a point. This is called promiscuous mode. Once the network adapter has done its job by connecting all packets through a point the software part needs to disassemble, log and reassemble the software packets.

Each packet sniffer can usually only connect to one channel at a time and miss packets moving though other channels so a packet sniffer is needed for each channel to be observed. The structure of the network also determines how many sniffers are needed, the way the network switches are configured determines if only a segment of the network is accessible.

To create a simple packet sniffer in python the help socket module is helpful. The raw socket type is used to get the packets. Other modules to use are struct, which allows unpacking and packing of strings according to a specified format.

A socket is a software structure within a network node that serves as an endpoint for sending and receiving data. The structure and properties of a socket are defined by the application programming interface (API). Sockets are created only during the lifetime of a process of an application running in the node, so they are specific to the application they are opened for.

A socket has 3 parameters, the first tells the packet interface (PF_PACKET for linux, AF_INET for windows); the second says it is a raw socket and the third is the protocol of the packet we are interested in (0x0800 for IP). The protocol numbers are designed by the Internet Assigned Numbers Authority (IANA). For AF_INET this refers to IPV4 so we must use IP protocols in the last parameter.

s = socket.socket(socket.AF_INET, socket.SOCK_RAW, socket.htons(0x0800))

To receive a packet we need to use a receive method from the socket in an infinite loop and the parameter is the max buffer size:

while True:

packet = s.recvfrom(2048)

Then we need to rip the ethernet header

ethernet_header = packet[0][0:14]

Then we can rip and unpack the ip header

ipheader = pkt[0][14:34]

ip_header = struct.unpack("!12s4s4s", ipheader)

print "Source IP:" + socket.inet_ntoa(ip_header[1]) + " Destination IP:" + socket.inet_ntoa(ip_header[2])

(if we want to view the ethernet header we must unpack it too)

eth_header = struct.unpack("!6s6s2s", ethernet_header)

print "Destination MAC:" + binascii.hexlify(eth_header[0]) + " Source MAC:" + binascii.hexlify(eth_header[1]) + " Type:" + binascii.hexlify(eth_header[2])

//APR: When a machine wants to communicate with another it must find the physical address by using the Address Resolution Protocol table and then convert the packet to the correct length to send it to the machine. If no entry is found then an APR request is sent to all machines on a network.//

## Data Packet Capture - Owen

https://www.thousandeyes.com/learning/glossary/packet-capture

https://www.varonis.com/blog/packet-capture#:~:text=What%20is%20Packet%20Capture%3F,-%C3%97&text=Packet%20Capture%20refers%20to%20the,pcap%20format

### What is Network Packet Capturing?

Capturing is the process for intercepting data packets travelling through a network.

After capture, it can be analysed and used to help determine network problems such as security threats and any lost packets.

After a data breach, these packets can provide forensic clues to help with investigations.

Packet Capture can also be a threat as they can be used to steal important information in transit, and capturing packets can be accomplished without leaving any trace.

Types of packet capture:
We can choose to capture entire packets or only specific sections of it such as just the header through filtering in order to save time, space, and resources.

### How does packet capturing work?

Packeting capturing involves creating copies of data packets passing through a specific point in a network.

This can be done from a router or a switch or any computer device.

Own machine:

Easiest method, however network interfaces only pay attention to traffic destined for them.

In order to get more network traffic, the interface needs to be put in promiscuous mode or monitor mode. This method makes packet capture limited, since you will only see traffic on the local switch port your machine is connected to on a wired network.

Router or switch:

Port mirroring, port monitoring allow network traffic to be copied and sent to a specific port, in order to export them to a dedicated monitoring system. Firewalls and WAPs also have packet capture methods.

Packets are captured going through a device in the network

## Analysis - Protocols - Owen:

In order to break down a packet, we need to know the contents and what they are used for.

### TCP and IP:

Internet Protocol (IP)

- Used for sending data across the internet or a smaller network, between source and destination addresses.

- Defines how applications and devices send packets.

- TCP/IP protocol stack is used to achieve this.

- IP obtains the IP address.

Transport Control Protocol (TCP)

- Responsible for data delivery once an IP address is found.

- Establishes a connection between the source and destination devices, and makes sure it stays active, until it ends.

- Breaks down the data into packets.

- High level protocol that transmit data use TCP, such as FTP, SSH, IMAP, POP, SMTP, and HTTP.

- UDP is an alternative but does not have packet sequencing or errors, however it is less expensive

### TCP/IP model:

- Default method of data communication across the Internet.

- Divides tasks into layers to keep processes standardised.

Four layers:

- Data Link Layer:

  - Defines how data should be sent, the physical process of exchanging data

- Internet Layer:
	Sending packets from a network and ensures they get to their destination.

- Transport Layer:

  - Establishes data connection between sender and receiver.

  - Divides data into packets and sequences them.

  - Determines where packets should be sent, at what rate etc.

- Application Layer

  - Level users interact with.

  - Combines sessions, presentation and application layers of the OSI model.

Data packets sent over TCP/IP are not private, and can be intercepted.

### OSI Model:

An alternative to TCP/IP is the Open Systems Interconnection (OSI) model, which consists of seven layers instead of TCP/IPs four.

TCP/IP vs OSI:

TCP/IP:

- Solves specific communication problems

- Uses specific, standard protocols

- Most applications use all 4 layers

OSI Model:

- Generic, protocol-independent

- Can describe all forms of network communication

- Simple applications only use the 1st,2nd, and 3rd layers sine they are the only ones required for data communication.

https://www.imperva.com/learn/application-security/osi-model/

OSI’s Seven Layers:

(Source: https://www.bmc.com/blogs/osi-model-7-layers/)

1 Physical:

- Communication of raw data over a physical medium.

2 Data Link

- Data transmission between network nodes

- Connections between connected devices

- Data from physical layer packaged into frames, containing protocols to send them between nodes

3 Network

- Organises and transmits data between multiple networks.

- Routes data through the optimum path.

- Addressing for packets to distinguish source and destination networks.

- Divides into packets, and put back together at receiving end

- IPv4, IPV6

4 Transport

- Ensures reliable delivery of packets

- TCP, UDP

5 Session

- Manages sessions between servers to coordinate communication,

- Responsible for starting, ending, authentication etc, of communication.

6 Presentation

- Converts data formats between applications and networks

  - Data conversion

  - Compression

  - Encryption

7 Application

- Layer that the user interacts with

### Contents of a IP packet header:

- Version - version number of Internet Protocol being used such as IPv4 or 6

- IHL - Internet Header Length - length of the entire IP header

- DSCP - Differentiated Services Code Point - type of service

- ECN - Explicit Congestion Notification; it carries information about the congestion seen in the route

- Total Length - length of the whole IP packet, i.e. header + payload

- Identification - if IP packet is fragmented during the transmission, all the fragments contain some identification number to identify the original packet they belong to

- Flags - required by network resources, if IP packet is too large to handle, these “flags” tells if they can be fragmented or not.

- Fragment offset - the exact position of the fragment in the original IP packet

- Time to Live - In order to avoid endless looping, packets have a TTL value, this tells the network how many routers, known as hops, this packet can cross decrementing the value until the packet is discarded.

- Protocol - tells the Network layer at the destination host, to which protocol this packet belongs to, e.g. protocol number of ICMP is 1, TCP = 6, and UDP = 17.

- Header Checksum - checksum value of entire header in order to check if the packet is received without errors

- Source address - 32-bit address of the source of the packet

- Destination address - 32-bit address of the destination of the packet

- Options - optional field used if value of IHL is larger than 5. Options contains values for options such as security, record route, time stamp, etc.

### Contents of a TCP Header:

Source: (http://books.gigatux.nl/mirror/securitytools/ddu/ch06lev1sec3.html)

IP header establishes connection with correct device, and the TCP header is used to establish a TCP session as to send the packets to the right ports on the established device, 20 Bytes long

- Source port - 16 bits

- Destination port - 16 bits

  - These port numbers identify which ports the packets neede ot be sent to.

- Sequence number - used to reassemble the packets in the correct order at the receiving end.

- Acknowledgement number - 32 bits - verification that it is coming from the correct device.

- Offset - 4 bits - number of 320bit lines there are in the TCP header

- Reserved - 6 bits - same as IP header, these are reversed in case needed in the future

- TCP flags - 6 bit -controls how the packet is handled

- Window size - amount of bits the sender is able to receive

- TCP checksum

- Urgent pointer

- TCP options - additional information - padded out to 32 bits

TCP Flags:

(Source: http://books.gigatux.nl/mirror/securitytools/ddu/ch06lev1sec3.html)

Apart from the header, packets also contain a payload which consists of the actual data being sent.

The process of analysing the payload is called Deep Packet Inspection (DP) and can identify malicious traffic and malware etc. inside, however we will aim to filter the payload out due to the considerable size of them and any privacy issues associated with it.

Problems to solve:

- Packet capture is easy for incoming packets to local machines by using python libraries such as socket, much harder to actively collect packets from a network.

- There are a variety of ways to analyse a data packet so will need to decide on the scope of our program.

- Need to choose which library to use for capturing and analysis

### Potential Python Libraries:

Socket:

- Sockets are used to send messages across a network.

- Provides access to a socket interface, which is transliteration of the Unix system call and library interface for sockets to Python’s object-oriented style.

- Need to break down received packets manually requiring knowledge of packet structure.

Scapy:

- External library

- Enables the user to send, sniff and dissect, and make network packages.

- Allows us to probe or scan networks.

- Suitable for a variety of protocols

- Scanning, trace routing, probing, unit tests, network discovery

Pyshark:

- External library

- Python wrapper for tshark (wireshark command-line utility)

## Network risks and attacks - Antonio:

## Database for accounts and admins - Chen:

Database: mysql (an opensource database) / other specify database (might be)

Python module: mysql.connector(develop by mysql developer) / pymysql(similar to the first one)

Implement:

·Store the data package

·Help the users to create a local database on their computer/server (might be needed)

·Store the historical data for some days (7days of 30days, then automatic clean-up)

·Store some meta data then provide the reference to us for analysis

We might get more information about database after the reading week:

## Choice of Programming Language:

Python:

Justification:

## Python libraries for NTA - Harvey

7 Layers of the OSI Model

(Source : https://www.bmc.com/blogs/osi-model-7-layers/)

>> Socket (Built-in library)

Official documentation: https://docs.python.org/3/library/socket.html

Usage : 1) Enables Python to communicate with Sessions

2 ) Make server and client socket interact each other

>> PyPcap (Python Packet Capturing Library) - External Library

Pypcap is a python wrapper of libpcap, which means pypcap enables us to use     libpcap in python programs.

Official documentation: https://pypcap.readthedocs.io/en/latest/

https://github.com/pynetwork/pypcap

Usage : 1 ) Sniff the packet and collects data such as MAC addresses

2 ) Parse the packet into IP and ethernet (allows to observe each)

3 ) Useful to keep track of capturing packets of ‘live’ network

>> Pyshark - External library

Pyshark is a python wrapper for tshark

Official documentation : https://github.com/KimiNewt/pyshark/